vishveshvariah technological university

department of medical electronics			[ML6L4]

digital electronics and microprossers lab question bank

- a). Conduct a suitable experiment to verify the truth table of rs and jk flip – flop (constructed by nand gates only).

b). convert a jk flip – flop into a d flip – flop.

- a). Conduct a suitable experiment to verify the truth table of a jk master - slave flip – flop using nand gates only.

b). Convert this into a mod 2 counter.

- a). Conduct a suitable experiment to perform the following operations using ic [7495/7496] siso, piso, sipo, pipo.

b). Using the same chip convert the circuit as a ring counter/twisted ring counter.

- Using 7493 IC chip design a modulus `N` counter (N will be specified by the examiner) and an experiment to verify the same.

- Write a program to move a data block starting at loc x (to be specified by the examiner) with/without overlap the loc X, Y as well as the block length as specified at suitable memory locations.

- Write a program to add 2 multibyte binary numbers stored at loc x and loc y and store the result at loc z and display the same in data/address field.

- Write a program to add 2 digit bcd numbers and store the result at loc x and display the same in data/address field.

- Write a program to add N one multibyte numbers stored at loc x +1 and n is stored at loc X. Store the result in loc y and loc y+1.  Display the same in data/address field.

- Write a program to subtract 16 bit numbers stored at loc x and loc x+1 from an another 16 bit number from loc y.

- Two unsigned binary numbers are stored at loc x and loc x+1 .Find the product and store the result in location x+2 and x+3 (if any carry is there). Display the same in address field.

- Divide a 16 bit number at loc x and x+1 by 8 bit number at loc Y; display the quotient in address field and the remainder in data field.

- Write a program to check whether a byte stored at loc x belongs to 2 out of 5 code. Display ff if it is 2 out of 5 code otherwise 00 in the data field.

- Write a program to convert a binary number stored at loc x to its bcd equivalent & display in the data field.

- Write a program to convert a BCD number stored at loc x to its binary equivalent & display in the data field.

- Write a program to convert an ASCII number stored at loc x to its binary equivalent & display in the data field.

- Write a program to convert a binary number stored at loc x to its ASCII equivalent & display in the data field.

- Write a program to simulate throw of a dice using interrupts

.

- Write a program to generate a triangular wave using dac & display the waveform on the cro, measure the slope.

- Write a program to generate a square wave of a given duty cycle using dac. Display the same on cro & verify the same.

- Write a program to implement a moving display of a given string of digits on a display interface with a suitable delay.

- Write a program to interface a 4*4 keyboard and display the code of key pressed.

- Write a program to evaluate a 3 variable multiple Boolean expression using logical controller interface (the expression to be given by the examiner).

Index

- TO MOVE a data block from one location to ANOTHER.

- addition of 2 multi byte binary numbers.

- addition of 2 bcd numbers.

- addition of n one byte binary numbers.

- subtraction of 16 bit numbers.

- multiplication of 2 unsignged numbers

- division of 16 bit numbers.

- to check 2 out of 5 CODE.

- conversion of binary to bcd equivalent.

- conversion of bcd to binary equivalent.

- conversion from ascii to binary equivalent.

- conversion from BINARY TO ascii equivalent.

- generation of triangular wave.

- geration of square wave.

- simulation of throw of a dice.

- implementation of moving display OF A given display.

- interfacing of a 4*4 keyboard matrix and display the code of key pressed.

- evaluation of 3 variable multiple Boolean expression using logic gate controller interface.

1. a) Write a program to move a data block starting at loc x (to be specified by the examiner) without overlap the loc X, Y as well as the block length as specified at suitable memory locations.

solution:

comments

lda 8900h 		Load accumulator direct

mov c, A 		Move contents of register A to C

lhld 8901h		Load H and L registers direct

xchg			Exchange H and L with D and E

lhld 8903h		Load H and L registers direct

loop 		mov a, M		Copy from M to A

stax d		Store accumulator direct

inx h			Increment register pair by HL by 1

inx d			Increment registers pair by HL by 1

dcr c			Decrement register C by 1

jnz loop		Jump on no zero to loop

rst 5			Restart

INPUTs:			comments

| Block length 8900 | 03 |
| --- | --- |
| 8901 | 50 |
| 8902 | 88 |
| 8903 | 70 |
| 8904 | 88 |

LOC y

LOC X

LOC X

| 8870 | 01 |
| --- | --- |
| 8871 | 02 |
| 8872 | 03 |

LOC Y

| 8850 | xx |
| --- | --- |
| 8851 | xx |
| 8852 | xx |

OUTput:

| 8850 | 01 |
| --- | --- |
| 8851 | 02 |
| 8852 | 03 |

1. b) Write a program to move a data block starting at loc x (to be specified by the examiner) with overlap the loc X, Y as well as the block length as specified at suitable memory locations

solutions:

comments

lhld 8A00h		Load HL pair with contents of 8A00.

LDA 8B00H		Load accumulator with contents of 8B00.

mov c, a		Move contents of A to C.

dcr c			Decrement contents of register C.

mvi B 00h		Move immediately 00 to register b.

dad b			Add contents of HL with BC.

xchg			Exchange contents of HL with DE pair..

lhld 8a02h		Load HL register with contents of 8A02.

dad b			Add contents of HL with BC.

inr c			Increment the contents of c register.

loop		move a, m		Move contents of memory to accumulator.

stax d		Store accumulator indirect.

Dcx h			Decrement contents of register H.

dcx d			Decrement contents of register D.

dcr c			Decrement contents of register C.

jnz loop		Jump on no zero to loop.

rst 5			Restart.

input:									output:

| 8b00 | 05 |
| --- | --- |

| 8a00 | 52 | LOC Y (D) LOC X (S) | 8850 | 01 |  | 8852 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8a01 | 88 |  | 8851 | 02 |  | 8853 | 02 |
| 8a02 | 50 |  | 8852 | 03 |  | 8854 | 03 |
| 8a03 | 88 |  | 8853 | 04 |  | 8855 | 04 |
|  |  |  | 8854 | 05 |  | 8856 | 05 |
|  |  |  | 8855 | xx |  |  |  |

- Write a program to add 2 multibyte binary numbers stored at loc x and loc y and store the result at loc z and display the same in data/address field.

solutions:

xra a			Exclusive OR with accumulator.

lxi d 8860h		load DE pair with 8860 immediately.

lxi h 8960h		load HL pair with 8960 immediately.

lxi b 8A60h		load BC register pair with immediately.

mvi a 04h		move 04 to accumulator immediately.

back		sta 8C00H		store accumulator direct.

ldax d		load accumulator indirect.

adc m		add register to accumulator with carry.

stax b		store accumulator indirect.

inx d			increment DE pair by 1.

inx H			increment HL pair by 1.

inx b			increment BC pair by 1.

lda 8c00h		load accumulator direct.

dcr a			decrement source by 1.

jnz back		jump on no zero.

adc a			add register to accumulator with carry.

stax b		store accumulator indirect.

call disp		unconditional subroutine call.

hlt			halt.

disp		mvi b 05h		move immediately 8-bit.

lxi h 8a60h		load HL pair immediately.

next		push b		push register pair BC onto stack.

push H		push register pair HL onto stack.

mov a, m		copy M to A.

sta 8ff1h		store accumulator direct.

shld 8fefh		store H & l register direct.

call 044ch		 unconditional subroutine call.

call 0440h		unconditional subroutine call.

call delay		unconditional subroutine call.

pop h			pop off stack to register pair HL.

pop b			pop off stack to register pair HL.

inx h			increment HL pair by 1.

dcr b			decrement source by 1.

jnz next		jump on no zero.

ret			Return from subroutine call unconditionally.

delay	mvi b 03h		Move immediate 8 – bit.

l2		lxi d ffffh		load DE pair immediately.

l1		dcx d			decrement DE pair by 1.

move a, d		copy from D to A.

ora e			logically OR with accumulator.

jnz l1			jump on non zero to L1.

dcr b			decrement source by 1.

jnz l2			jump on non zero to L1.

ret			return from subroutine unconditionally.

INPUT:								OUTPUT:

LOC X: I MULTIBYTE NO.	    LOC Y: II MULTIBYTE NO.

| 8860 | ee |  | 8960 | 6f |  | 8a60 | 5d |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 8861 | 77 |  | 8961 | d0 |  | 8a61 | 78 |
| 8862 | 56 |  | 8962 | 50 |  | 8a62 | a6 |
| 8863 | 48 |  | 8963 | e0 |  | 8a63 | 28 |
|  |  |  |  |  |  | 8a64 | 01 |

- Write a program to add 2 digit (8 - bit) bcd numbers and store the result at loc x and display the same in data/address field.

solutions:

lda 8900h		load accumulator direct.

mov b, a		copy contents of A to B.

lxi h 8901h		load HL pair directly.

mvi a 00h		move immediately 8 – bit.

mov c, a		copy contents of A to C.

l2		add m		add register to accumulator.

daa			decimal adjust accumulator.

jnc l1		jump on non carry to L1.

mov D, A		copy from A to D.

mov A, C		copy from C to a.

adi 01h		add to immediate to accumulator.

daa			decimal adjust accumulator.

mov C, A		copy from A to C.

mov a, D		copy from D to A

l1		inx h			increment HL pair by 1.

dcr b			decrement source by 1.

jnz	l2		jump on non zero to l2.

sta 8fefh		store accumulator direct.

mov a, c 		copy from C to A

sta 8ff0h		store accumulator direct.

call 0440h		unconditional subroutine call.

hlt			halt.

input:

| 8900 | 03 |
| --- | --- |
| 8901 | 50 |
| 8902 | 60 |
| 8903 | 70 |

output:

| 01 | 80 |
| --- | --- |
| 8ff0 | 8fef |

input:

| 8900 | 03 |
| --- | --- |
| 8901 | a0 |
| 8902 | 0b |
| 8903 | bc |

output:

| invalid 01 | 50 |
| --- | --- |
| 8ff0 | 8fef |

- Add N 1 byte binary numbers stored from LOC X and N stored at LOC X . Store the result in data/ add field.

xra a			Exclusive OR the contents of the reg with the accumulator.

Mvi b 00h		Store 00 in B.

lhld 8a00 		Load 00 in register L & 8A in register C.

mov c, M		Store contents of memory in register C.

inx h			HL registers is incremented by 1.

up	add m		Add the contents of HL register pair to Accumulator.

jnc next		Jump on no carry to Next.

inr b 			Increment the contents of B by 1.

next	inx h			Increment the contents of HL register by 1.

dcr c			Decrement the contents of C by 1.

jnz up		Jump on no zero to UP.

mov l, A		Move the contents of accumalator to L.

mov h, B		Move contents of B to H.

shld 8fefh		Store the contents of HL register to memory 8FEF.

call 0440h		Call the instruction at memory location 0440H.

hlt			Halt further execution of the program.

INPUT:			OUTPUT:

| 8a00 | 04 | blk length | 00 | a0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 8801 | 10 |  | 8ff0 | 8fef |  |  |
| 8802 | 20 |  |  |  |  |  |
| 8803 | 30 |  |  |  |  |  |
| 8a04 | 40 |  |  |  |  |  |

- Subtract 16 bit number stored at LOC X and LOC X+1 from another 16 bit number at LOC Y , LOC Y+1 .Store the result at LOC Z , LOC Z+1 and display the result.

LHLD 8850H		Transfers memory contents to H register pair.

XCHG			Contents of HL pair are exchanged with DE pair.

LHLD	8950H 		Transfers memory contents to HL register pair.

MOV A, L		Store contents of register pair in accumulator.

SUB E		The contents of register E are subtracted from the  Acc.

MOV L, A 		Store the contents of A in L.

MOV L, H		Store the contents of H in A.

SBB D			Reg D Contents are subtracted with borrow from Acc.

MOV H,A 		Move contents of accumulator to register H.

SHLD 8A00 H		Store the contents of register HL in memory 8A00H.

SHLD 8FEF H	Store the contents of register HL in 8FEF H.

CALL  0440 H		Call the instruction of memory location 0440 H.

HLT			Halt the execution of the program any further.

Example:

78C2 H - 23F3 H  = 54CF H

INPUT:

LOC X						LOC Y

| X | 8850 | F3 |  | Y | 8950 | C2 |
| --- | --- | --- | --- | --- | --- | --- |
| X+1 | 8851 | 23 |  | Y+1 | 8951 | 78 |

OUTPUT:

| 8A00 | CF |  | 54 | CF |
| --- | --- | --- | --- | --- |
| 8A01 | 54 |  |  |  |

6. Multiplication of two unsigned bits.

LXI H, 0000 H		Loads 00 in register H and L.

LDA 8850 H 		Loads the Acc. with the contents of mem location  8850.

CPI 00 H		Compare 00 H with the Acc. Contents.

JZ	EXIT 			Jump on zero to exit .

MOV E, A		Store the contents of Acc. in Register E.

LDA 8851 H		Loads the Acc. with the contents of 8851 H location.

CPI 00 H		Compare 00 with Acc. contents.

JZ	EXIT			Jump on zero to exit.

MOV C, A		Store the contents of Acc. in C.

MVI D, 00 H		Move 00 immediately into register D.

LOOP	DAD D		Add the contents of DE Register  pair with contents

of HL and store in HL register pair.

DCR C			Decrement register C pair by 1.

JNZ	LOOP			Jump on no zero to Loop.

EXIT 	SHLD 8FEF		Store the contents of register HL in the memory loc. 8FEF.			CALL 0440 H		Call the instruction at memory location 0440.

HLT			Halt any further execution of the program.

Example :

02 H * 05 H

INPUT:

| 8850 | 02 |
| --- | --- |
| 8851 | 05 |

OUTPUT:

| 8FF0 | 00 |
| --- | --- |
| 8fEF | 0A |

7. Program to divide a 16 bit number.

LHLD 8A00 H		Transfers memory contents to HL Register.

LDA 8A02 H		Loads the Accumulator with contents of

8A02.

MOV E, A		Move contents of accumulator to register E.

MVI D 00 H		Move 00 immediately into register D.

LXI B 0000 H		Loads BC register pair with 00.

BACK		MOV A, L		Store the contents of L in accumulator.

SUB E			Subtract the contents of E from Acc.

MOV L, A		Store the contents of accumulator in L.

MOV A, H		Store the contents of H in accumulator.

SBB D			Subtract D with borrow from accumulator.

MOV H, A		Store the accumulator contents in H.

JC		EXIT			Jump on carry to EXIT.

INX B			Increment BC register pair by 1.

JMP 		BACK			Jump unconditionally to BACK.

EXIT		DAD D		Add DE register pair to HL and store result

in HL pair.

MOV A, L		Move contents of L in A.

STA 8FF1 H 		Store accumulator contents in location

8FF1.

MOV L, C		Store contents of C in L .

MOV B, H		Store contents of H in B.

SHLD 8FEF H	Store contents of HL pair in 8FEF and

8FF0.

CALL 044C H		Call the instruction at 044C.

CALL 0440 H		Call the instruction at 0440.

HLT			Halt any further execution of the program.

Example :

0145 H  ÷ 0C H

INPUT:

| 8A00 | 4F |
| --- | --- |
| 8A01 | 01 |
| 8A02 | 0C |

OUTPUT:

| 00 | 1b |
| --- | --- |
| 8ff0 | 8fef |

01

8ff1

REMAINDER					QUOTIENT

8. Write a program to check whether a byte stored at LOC X belongs to 2 out of 5 code.

LDA 8900H	Loads the accumulator with contents of 8900.

MOV B,A	Move contents of accumulator in Register B.

ANI E0 H	And immediate with accumulator.

JNZ		INVALID 	Jump on no zero to invalid .

MOV A, B	Store contents of B in accumulator.

MVI D 00 H	Move 00 into D.

MVI E 05 H	Store 05 in register E.

LOOP		RRC		Rotate accumulator right.

JNC		NEXT		Jump on no carry to next.

INR D		Increment the contents of register D by 1.

NEXT		DCR E		Decrement register E by 1.

JNZ		LOOP		Jump on no zero to loop.

MOV A, D	Store the contents of D in A.

CPI 02 H	Compare immediate  with accumulator.

JNZ 		INVALID	Jump on no zero to invalid.

MVI A, FF H	Store FF in accumulator.

JMP		DISP		Jump unconditionally to disp.

INVALID	MVI A, 00 H	Store 00 in accumulator.

DISP		STA 8FF1 H	Store accumulator contents in 8FF1.

CALL 044C 	Call instruction at memory location 044C.

HLT		Halts any further execution of the program.

Example:

1. Input :

- 03

output :

8FF1   FF		/*  03 is 2 out of 5 code  */

2. input:

- 15

output :

8FF1    00		/*  15 is not 2 out of 5 code  */

9. Write a program to convert a binary number to its BCD equivalent.

LDA 8900 H	Stores the contents of 8900 in accumulator.

MOV C, A	Store accumulator contents in register C.

LOOP 		LXI H, 0000	Load HL register pair with 00 00.

CPI  64 H	Compare immediate with accumulator.

JC 		FRWD		Jump on carry to forward.

SUI 64 H	Subtract 64 from accumulator contents.

INR H		Increment register H by 1.

JMP 		LOOP		Jump unconditionally to loop.

FRWD		CPI 0A H	Compare immediate with accumulator.

JC 		FRWD1	Jump on  carry to FRWD1.

SUI 0A  H	Subtract 0A  from accumulator contents.

INR L		Increment register L by 1.

MOV B, A	Store the contents in A in B.

JMP		FRWD		Jump unconditionally to FRWD.

FRWD1	MOV A, L	Store the contents of L in A.

RLC		Rotate accumulator right.

RLC		Rotate accumulator right.

RLC		Rotate accumulator right.

RLC		Rotate accumulator right.

ADD B		Add contents of B to A.

MOV L, A	Move contents of A to L.

SHLD 8FEF 	Store HL contents in 8FEF & F0.

MOV A, C	Store contents of C in A.

STA 8FF1 H	Loads contents of accumulator in 8FF1.

CALL 0440	Call instruction at 0440.

CALL 044C	Call instruction at 044C.

HLT 		Halts any further execution of the program.

Example :

Input:

- 0A

Output:

00		10		0A

8FF0	8FEF		8FF1

11. Write a program to convert a BCD number to its binary equivalent.

LDA 8900H		Load Accumulator with contents of 8900H.

MOV B, A		Store contents of Accumulator in B.

ANI 0FH		AND immediate with Accumulator.

MOV C, A		Store the contents of Accumulator in C.

MOV A, B		Store the contents of B in Accumulator.

ANI F0H		AND immediate with Accumulator.

JZ		DN			Jump on zero to DN.

RRC			Rotate Accumulator right.

RRC			Rotate Accumulator right.

RRC			Rotate Accumulator right.

RRC			Rotate Accumulator right.

MOV E, A		Store the contents of Accumulator in E.

XRA A			Exclusive OR the contents register of register.

MVI H 0A		Move immediately 0A to register H.

LOOP		ADD H 		Add contents of H to Accumulator.

DCR E 		Decrement the register E by 1.

JNZ 		LOOP			Jump on no zero to loop.

DN		ADD C 		Add contents of C to Accumulator.

STA 8FF1H		Store contents of Accumulator in 8FF1.

CALL 044CH		Call instruction in 044C.

HLT			Halt any further execution of program.

EXAMPLE:

1. input

- 09

output

8FF1	09

2. input

- 34

output

8FF1	22

11. Write a program to convert an ASCII number to its Binary equivalent.

LDA 8850H		Loads contents of the Accumulator in 8850.

SUI 30H		Subtract 30 from contents of the Accumulator.

CPI 0AH 		Compare immediate with 0A.

JC		L1			Jump on carry to L1.

SUI 07			Subtract 07 from the contents of Accumulator.

L1		STA 8FF1 H		Loads the Accumulator contents in 8FF1.

CALL UPDDT		Call instruction at memory location 044C.

HLT			Halts any further execution of the program.

Example:

1. Input:

    - 41

Output:

8FF1	0A

2. Input:

- 39

Output:

8FF1	09

12. Write a program to convert a binary number to its ASCII equivalent and display.

LDA 8850	Loads Accumulator with the contents of 8850.

ANI 0FH	AND immediate with Accumulator.

CPI 0A H	Compare immediate with 0A H.

JC		NEXT		Jump on carry to NEXT.

ADI 07H	Add 07 to Accumulator.

NEXT		ADI 30H	Add 30 to Accumulator.

STA 8FF1	Store the contents of Accumulator in mem. location 8FF1.

CALL UPDDT Call instruction in accumulator to 044C address.

HLT		Halts any further execution of the program.

EXAMPLE:

1. INPUT:

- 0A

OUTPUT:

8FF1	41

2. Input:

- 09

Output:

8FF1	39

13. Write a program, to generate a triangular wave using DAC. Display the waveform on CRO and measure the slope.

Theory:

Port address of 8255

Port A	- 40

Port B	- 41

Port C	- 42

Control Word:

| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

BSR-0					Port A			Mode			Port B

Y

X

Slope:  Y/X

MVI A 80	Move immediately 80 into the Accumulator.

OUT 43H	Output data from Accumulator into the port specified.

XRA A		Exclusive OR contents of A with Accumulator.

LOOP1	OUT 40H	Contents of Accumulator are copied to port A.

INR A 		Increment A by 1.

CPI FF	Compare the contents of Accumulator with FF.

JNZ		LOOP1	Jump on no zero to LOOP1.

LOOP2	OUT 40H	Contents of Accumulator are copied to port A.

DCR A		Decrement A by 1.

JNZ 		LOOP2	Jump on no zero to loop2.

JMP 		LOOP1	Jump unconditionally to Loop1.

14. Write a program to generate a square wave of given duty cycle using DAC. Display the wave forms on the CRO and verify the same.

For 50% duty cycle

MVI A80 H		Move immediate 80 to Accumulator

OUT 43 H		Output data from Accumulator to the control reg.

MVI A00H		Move 00 into Accumulator.

OUT 40H		Output data from Accumulator to port A.

CALL DELAY		Call instruction in accumulator to delay.

MVI A FFH		Move immediate FF to Accumulator.

OUT 40 H		Output data from Accumulator to port A.

CALL DELAY		Call instruction in accumulator to Delay.

JUMP 		UP			Jump unconditionally to up.

DELAY	LXI B 003H		Store 3F in register and 00 in B register.

UP1		DCX B			Decrement BC pair by 1.

MOV A, C		Copy the contents of C in accumulator.

ORA B			OR the contents of accumulator.

JNZ		UP1			Jump in no zero to UP1.

RET			return from sub routine.

Theory:

The count value is calculated as follows:-

Total T states = [10+ (6+4+4) count -3+10]

=17+24*count.

Frequency of clock out signal of the processor = 3.0724

Total delay = 17+24 * count

Count= (0.5 * 10-3 * 3.072 * 106 - 17) / 24

= (63.29)10

Count= (63)10= (3F) 16

% Duty cycle = (Ton) / (Ton+ Toff).

14. Write a program to stimulate a throw of dice using interrupts.

MVI A 08 H 		Load 8 bit word into accumulator.

SIM			Set interrupt mask.

EI			Enable interrupts.

UP1		MVI A 01 H		Load 01 immediately into accumulator.

UP		INR A 			Increment the value of contents of accumulator.

CPI 06 H		Compare 06 immediately with accumulator.

JNZ		UP			Jump on no to zero to UP.

JMP 		UP1			Jump unconditionally to UP1.

JMP 		8850			Jump unconditionally to address location.

PUSH PSW		Push onto stack.

STA 8FF1 H		Store contents of Acc. in 8FF1 address location.

CALL UPDDT		Call instruction at UPDDT.

POP PSW		Pop it from stack.

EI 			Enable interrupt.

RET			Return from subroutine unconditionally.

15. Write a program to implement a moving display of a given string of digits on display.

MVI A 80 H 		Load 80 into accumulator.

OUT 43 H 		Output data from accumulator to control register.

L3		LXI H 8900  		Load 00 in H and 89 in L register.

MVI B 04 H		Move immediately 09 to register B.

L2		MVI C, 08		Move 08 to register C.

MOV A, M		Move contents of memory to accumulator.

INX H 			Increment contents of register B by 1.

L1		RLC			Rotate accumulator left.

MOV E, A		Move the contents of A to A.

OUT 41 H 		Output data from accumulator to port B.

MVI A 01 H		Move 01 to accumulator.

OUT 42 H		Out put data from accumulator to port C.

DCR A			Decrement contents A by one.

OUT 42		Out put data from accumulator to port C.

MOV A, E 		Move the contents of register E to A.

DCR C			Decrement the contents of register C by one.

JNZ L1					Jump on non zero to L1.

CALL DELAY 		Call instructions in accumulator to DELAY.

CALL DELAY 		Call instructions in accumulator to DELAY.

DCR B 		Decrement B by one.

JNZ L2					Jump on non zero to L2.

JMP	L3				Jump to L3.

DELAY	LXI D FF FF		Load FF in D and E.

BACK		DCX D			Decrement DE pair.

MOV A, E				Move contents of E to A.

ORA D			OR the contents of D with accumulator.

16. Write a program to interface a 4*4 keyboard Matrix and display the code if key pressed.

MVI A 90 	Move 90 to accumulator .

OUT 43	output data from accu,mulator to control register.

START 	MVI A 07	Move 07 to Accumulator .

OUT 42	Output data from accumulator to Port C.

WAIT 		IN 40 		Input data from accumulator to port A.

CPI 00		Compaer 00 with contents of acckumulator .

JZ 	WAIT		Jump o zero to wait.

MVI B 00		Move 00 to register  B.